CITIZEN COOPERATIVE BANK LTD., VIJAYPUR,

| S.NO. | DESCRIPTION OF ITEM | QUANTITY | RATE | AMOUNT |
| --- | --- | --- | --- | --- |
| 1. | FRONT LINE COUNTER & CASH COUNTER |  |  |  |
| 2. 3 4. 5 6 7. 8. 9. 10 11 12 13. 14 | Providing & Fixing in position Counters as per Layout Plan in single/double level 2'-6",4’-0” high & 2'-6" deep.Counter shall be made of 19mm thick (ISI marked) Phenol bonded Plywood ( Century/Green/ Truwood) with 1mm thick laminate (Formica Century/Green/Heritage) on visible surface in combination of shades ( Grey & Blue) as per Architect approval. Front apron will be in single/ double level, finished with laminate with appropriate grooves with provision of cable management for computer cables at appropriate place, with lipimg 3/4"x1/4" on inner side and all edges including bottom touching floor and half round moulding 3.4"x3.4" on inner sides (public sides). The counter shall have keyboard tray having size approx 22"X15" or as directed made of 19mm thick plywood with 1-1/2" X 3/4" wooden moulding front and back sides with champhred edges and 3/4" X 1/4" liping on remaining two sides and 1mm thich laminate on horizontal surface with drawer channel of 15" length. A fill depth CPU shelf 14" wide shall be provided at left hand side of counter underneath the working top, at 4"above the floor level and 3" away (horizontally) from vertical ends (officer's side )which will be made of 19mm thick plyboard laminated with 1mm thick laminate on top & 3/4" X 1/4" wood lipping on two visible sides. A full length foot rest made of hard wood 3" X 1-1/2" section shall be provided at 4"above floor level. Drawer unit of 16” wide having drawers and 6" clear shall be provided on right hand side of the counter, of which three sides will be made of 12mm commercial plywood and front side of 19mm plyboard with 1mm thick laminate. The drawer will be provided with handle/knob and Lock of good quality to be got approved before fixing Drawer fitted on telescopic channel of 18" length. The drawer will be divided into six compartments with 12mm thick plywood. One cupboard with shutter, handle/knob/ auto closing lock and magnetic catcher can be considered in place of two drawers, if required. Single height 2’-6” Cash counter double height 4’-0” 12MM GLASS IN FRONT AND SIDE OF COUNTERS. Providing and fixing 12mm thick plain glass in front of cash counter with acid wash/etching of semi circular or any other pattern/ texture fixed to steel holder/ 38mm thick board having mica on both sides at appropriate distance sufficient in number so as to hold glass complete in all respects. All the visible edges of glass shall be grinded & polished beveled. The rates shall be inclusive of cost of all fitting, beadings, making suitable openings for transaction and voice communication etc. complete. WOODEN PARTITIONS/PENELLING/CASH CABIN Providing and fixing wooden partition 4'-6”&7'& upto ceiling level of high made of 3"X 1-1/2" hard wood frame @2'-0" c/c vertically & horizontally partitions of side of counters, the size of wood frames should be 3"x1-1/2.The wood frame should be fixed rigidly at floor level. The frame work to be cladded with 8mm thick (ISI marked) phenol bounded commercial ply fitted with both side with 1mm thk laminated on both sides as per architect direction and approved shade and colour & glass 5mm thick as per architect direction. All exposed fitted wood moulding (2-1/2" X1-1/2") & beading shall be stained to approve colour and finish with melamine polish or enamel painted. OFFICERS TABLE P/F wooden table of size 5'-0"X 2'-6" & 4’-6”X2’-3” made up of 19mm thick (ISI Marked phenol bonded boiling water proof plyboard with 1mm laminate on vertical asa well as horizontal surfaces with provision of 3" dia round cable management for computer cables at appropriate place with wood lipping 3/4"X 1/4" on inner side (officer side ) and all edged including bottom touching floor and half round wood moulding 3/4"X3/4" on remaining three sides. The top will be having 2-1/2" X 1-1/2" wood molding on all four sides. The table top will be protruding out 3" from sides & 9" from visitor's side. The outer 3" edges will be made of double board. Table has one side rack of 36"X15"X30" hanging two drawer & cabinet with shutter having good quality racks, magnetic catcher, hings, handle/knob etc on the right or left sides of the table. All the three sides of the drawer will be made of 12mm ply board and front side of 19mm ply board with 1mm thick laminated. The bottom of the drawer shall be made of 6mm thick ply board. The drawer will be provided with handle/ knob of good quality to be got approved before fixing. The front apron will be of teo levels with 6" frame from sides & 9" from top, 3/4" up them the base finished with laminate and sides to be finished with 3/4" X 1/4" wood lipping laminate at the base (front side) will be pasted in grooves. The table shall have wooden grey coloured key board tray having size approx. 22"X15" or as directed, made of 19mm thick (ISI marked) plyboard with 1-1/2"X3/4" wood moulding on front and back side and 3/4" X1/4" lipping on remaining two sides and 1mm thick laminate on horizontal surfacefixed on drawer channel of 15" length. A full deep CPU shelf 15"widw shall be proved at left or right hand side underneath the working top, at 5-1/2"above floor level and 3" away ( horizontally) from vertical end (officer's side) with 19mm thick plyboard laminated with 1mm thick laminate on top and 3/4"X1/4" nuwood lipping on visible edges. Afull length foot rest made of hard wood 3"X1-1/2" section shall be provided at 4" above floor level. Non laminated surface i.e. inside leg space of table/ inside of drawer/ cupboard/outside of drawer/ footrest/moulding shall be enamel painted. a) B.M. table 5’-0”x2-6” a) Officers. table 4’-6”x2-3” SOFA / SEATEE P/F two/ three seater sofa/ seatee with made up of solid local wood frame with MS heavy duty springs & jite filling duly covered with resin cloth 040 density polyurethane foam 4" thick for seat and 3" thick for back duly finished with tapestry cloth of approved shade, colour and design casting. The rate dhould be inclusive of all other material required for making standard design of sofa/ seater. NOTICE BOARD Providing and fixing notice board of size 2'X3' having 12mm ply fitted with 12mm soft board with cloth (velvet) of approved shade and having wooden beeding 1'-2"X3/4" in all sides dully painted. DOOR CLOSERS Providing and fixing hydraulic door closer ISI marked to doors complete in all respects. CENTRAL TABLE P/F central table of size 2'x2'x18" high with 12" thick glass top with beveled edges with base of 18" dia and made of 3/4" thick phenol bonded ISI marked plyboard with 1mm thick laminate as per design. VOUCHER STAND P/F Wall mounted vouchers stand 4’x1-6”x1’-6” and along the wall or required length as per site conditions having a writing top of 12mm thick glass base 19mm plyboard laminated in 1mm thick laminate of approved make and as detailed as per the architectural drawing plyboard with cut and exposed edges to have teak wood beading/lipping and spirit polished to the satisfaction of the Architect. ALUMINIUM WORK Aluminium door-cum- fixed windows Providing and fixing anodized aluminum door-cum-windows consisting of 14 guage / 2mm thick aluminum section made of 101.60x44.45mm and bottom rail of 114x44.45mm with 10 micron mat anodized manufactured by M/s. Volco extrusion limited or jindal finished as specified in drawings. door-cum- windows shall have 8mm thick plain tinted glass, fixed on aluminum beading with rubber gasket, screws, cleats etc complete in all respect. rates will include glazed door shutters with mat anodized aluminum frame with aluminum handles and locking arrangements as per drawings with or without screws as the case may be glazing clip with neoprene air tight gaskets lining fixed around frame of glass including cutting and marketing good the floor with cement, holes in the cement concrete / RCC all complete as per drawing and as per direction of Architect/ Bank Engineer including cleaning of original anodized finish on completion in all respects. Main door with 12mm thick glass patch stainless steel fitting and glass dully acid washed with design included heavy duty ISI marked. PAINTING Providing & Finishing walls with Plastic emulsion of approved brand and manufacture two or more coats over wall putty 2 coats complete including sand papering and cleaning the surface and cost of all materials, labour, machinery, scaffolding, all leads, lifts, and including cost of sampling, testing and water arrangements complete as per specifications and as per directions of the Architects. FALSE CEILLING P/fixing 2’-0" x 2'-0" perforated Armstrong made Calcium Silicate tiles for false ceiling. Installation: To comprise main runner spaced at 1200 mm centers securely fixed to a structural soffit by approved hangers at 1200 max. Centers and not more than 150 mm from spliced joints. last hangers at the end of each main runner should by greater than 600 mm from the adjacent wall. flush fitting 1200 mm long cross longer than 600x600 mm imitate to be formed by fitting 600 mm long flush fitting crossties centrally between the 1200mm crossties of ISI brand. DRESSING TABLE Providing & fixing of 5mm thk.Looking Glass on 19 mm thk plyboard size 2’-6”x 6’-0” fixed with fitting and drawers as per detail . STEPS Providing steps made up of 19mm thk plyboard with Riser 8”, tread 12” & size 2’-0” fited in three steps. | 16 Rft. 9 Rft 61 sft 313 sft - 1no. - 2nos. 12Seater. 2nos. 4no. 3nos 1no. 100 sft 2600 sft 1400sft 1no. 1no. |  |  |